- Open your desktop or laptop, locate the RAM slots, and remove and re-insert the RAM module to simulate reseating — then document each step you took and any issues you observed during the process.

- Shut down the computer completely.

- Unplug the power cable and disconnect all external devices.

- Press and hold the power button for about 10 seconds to discharge any remaining power.

- Open the computer case (or remove the laptop's service panel, if applicable).

- Locate the RAM slots on the motherboard.

- Push the retaining clips at both ends of the RAM slot outward.

- Carefully remove the RAM module by holding it at the edges. Avoid touching the gold contacts.

- Inspect the RAM module and slot for dust, dirt, or damage.

- Align the notch on the RAM module with the key in the RAM slot.

- Firmly press the RAM module straight down until both retaining clips snap into place.

- Close the computer case and reconnect the power cable and peripherals.

- Turn on the computer and verify that it boots normally and detects the installed RAM.

### Observations

- The RAM module was removed and reinserted successfully.

- The retaining clips locked into place after the module was fully seated.

- No visible dust, corrosion, or physical damage was observed on the RAM module or slot.

- The computer started normally after reseating the RAM.

- The BIOS and operating system detected the correct amount of installed memory.

### Possible Issues That May Be Observed

- RAM not fully inserted, causing the PC to fail POST or produce beep codes.

- Dust or debris preventing proper contact.

- Retaining clips not locking into place.

- Bent or damaged RAM slot or module.

- Computer displaying less RAM than installed if the module is not seated correctly.

- reate a troubleshooting checklist for diagnosing a faulty GPU in a gaming PC, including at least 5 steps from initial symptom identification to confirming the fix.<br><br><em><strong>Hint:</strong> Think about display issues, driver checks, and physical connections.</em>

| Step | What to Check | Expected Result / Action |
| --- | --- | --- |
| 1. Identify the Symptoms | Observe the problem (no display, screen flickering, artifacts, game crashes, blue screen, or poor performance). | Note when the issue occurs (startup, gaming, or idle). |
| 2. Check Physical Connections | Ensure the monitor cable (HDMI/DisplayPort) is securely connected to the GPU, verify PCIe power connectors are plugged in, and confirm the GPU is fully seated in the PCIe slot. | Reconnect any loose cables or reseat the graphics card if necessary. |
| 3. Check GPU Drivers | Open Device Manager or the GPU control software and verify the graphics driver is installed correctly. Update or reinstall the latest driver if needed. | The GPU should be recognized without warning symbols, and the latest driver should be installed. |
| 4. Test GPU Operation | Monitor GPU temperatures while gaming or running a stress test. Check for overheating, artifacts, crashes, or abnormal fan behavior. | GPU |

- Simulate a storage issue by unplugging and replugging a SATA or NVMe drive (SSD/HDD) on a test system, then describe the error messages or BIOS indicators you see before and after reconnecting.

| Stage | BIOS/PC Behavior | Typical Error Message or Indicator |
| --- | --- | --- |
| Before reconnecting (drive disconnected) | BIOS does not detect the SSD/HDD. Boot order may show no boot drive. | "No Boot Device Found", "Operating System Not Found", "Reboot and Select Proper Boot Device", or the SATA/NVMe port shows Not Detected. |
| After reconnecting (drive connected) | BIOS detects the drive by its model number and capacity. The drive appears in the storage device list and boot order. | No error message. The operating system boots normally if the drive is healthy. |

- Given a scenario where a PC randomly restarts, list which component (RAM, GPU, storage) you would check first and explain your reasoning, then outline the specific diagnostic steps you would perform for that component.

### Diagnostic steps for RAM

- Power off the PC

  - Shut down the computer and unplug the power cable.

  - Press the power button for a few seconds to discharge any remaining power.

- Reseat the RAM

  - Open the case and remove each RAM module.

  - Clean the contacts gently if needed and firmly reinstall the modules until the clips lock into place.

- Test one module at a time

  - If multiple RAM sticks are installed, boot the PC with only one stick.

  - Repeat the test for each module and, if possible, in different memory slots to identify a faulty stick or slot.

- Run a memory diagnostic

  - Use Windows Memory Diagnostic or MemTest86.

  - Allow the test to complete at least one full pass (multiple passes are recommended for better accuracy).

- Check BIOS settings

  - Verify that the RAM is running at the correct voltage and speed.

  - Disable any overclocking or XMP/EXPO profile temporarily to see if the system becomes stable.

- Monitor system stability

  - Use the PC normally or run a stress test after reseating and testing the RAM.

  - If the random restarts stop, the issue was likely related to faulty or improperly seated memory.